Problems For practice

Adiabatic Process

- A certain perfect gas is contained in a perfectly insulated piston cylinder arrangement at 100 kPa pressure and 290 K temperature. When the gas undergoes a non flow reversible compression process to 500 kPa and temperature rises to 350 K. If the workdone on the gas during compression is 50 kJ/kg.Evaluate the adiabatic exponent, specific heat at constant volume, characteristic gas constant and molecular mass of gas.

- 3 kg of air is kept at an absolute pressure of 100 kPa and 300 K temperature is compressed polytropically until pressure and temperature becomes 1500 kPa and 500 K .Evaluate the polytropic exponent n, final volume, work of compression and heat interactions

(R = 287 J/kgK, adiabatic index 1.4)

- 60 liters of an ideal gas is at 290 K and 1 bar initially is compressed adiabatically to 10 bar. It is then cooled at constant volume and further expanded isothermally to reach the condition where it is started. Evaluate

- Pressure at the end of constant volume cooling

- Change in internal energy during constant volume process

- Net work done and heat transfer to the cycle